衛武營國家藝術文化中心

- Data from 建築師雜誌

衛武營國家藝術文化中心位於高雄鳳山，過去是軍事新訓中心，後轉為都會公園，2003年劃定為47公頃的藝文園區。衛武營國家藝術文化中心基地面積9.9公頃，建物投影面積3.4公頃，是亞洲最大規模的表演場館。基地氣候是是高濕度與高鹽度的空氣，因此建築材料採用高雄的造船工業，並利用造船技術打造出獨特的鋼板曲面。經過精密計算後，兩千多片厚重但彎曲的鋼板被鑄造並焊接完成，最後成為半戶外榕樹廣場。

建築設計以園區內茂盛的榕樹群出發，創造通透、開放且內外合一的空間。民眾能自由地穿梭於表演場地與像室內又像戶外空間的榕樹廣場之間，使藝術能落實於日常生活裏。表演空間共有五個，包含：2,236座位的歌劇院、1,981座位的音樂廳、1,210座位的戲劇院、434個座位的表演廳，以及可容納多達五千名觀眾的戶外表演空間。館內還有亞洲最大的音樂廳管風琴。音樂廳採用最佳聲響的葡萄園式設計，以矮牆分隔階梯區塊，縮短聲響反射距離，使廳內每位聽眾都能享受最佳聲效。另外，部分聲學敏感區位採用盒中盒的概念，在主結構體間再加裝防震彈簧與隔震橡膠，有效阻隔外界雜音的傳遞。

衛武營是麥肯諾建築師事務所最具野心的建築之一，體現了團隊始終追求的建築理念。在這個以榕樹為意象的建築中，建築特殊的結構與外觀創造出流動又寬敞的自由空間。

- Data from archdaily

- 審稿編輯 María Francisca González

高雄市， 臺灣

- 建築師： Mecanoo

- 面積：141000 m²

- 專案年份：2018 年

- 攝影師：Iwan Baan

- Lei Bie： 演藝中心， 視覺藝術中心

- 本地建築師： Archasia Design Group

- 客戶： Preparatory Office of The Wei-Wu-Ying Center for the Arts of the Ministry of Culture， Taiwan

- 創始合夥人及創意總監： Francine Houben

- 藝術及執行董事： Chien Wen-Pin

- 結構工程師： Supertech

- 機械工程師： Yuan Tai

- 電氣工程師： Heng Kai

- 聲學顧問： Xu-Acoustique

- 劇院系統： Waagner-Biro

- 劇院顧問： Theateradvies

- 照明顧問： CMA Lighting Design

- 消防安全顧問： Ju Jiang

- 風琴顧問： Olivier Latry

- 屋頂立面顧問： CWI

- 3D 顧問： Lead Dao

- 交通系統顧問： Su International

- 承包商： Chien Kuo Construction

- 鋼外殼： Centraal Staal

- 風琴匠： Klais， Bonn

- 城市： 高雄市

這座壯觀的國立高雄藝術中心是由荷蘭 Mecanoo 建築事務所設計，其靈感來自於該地區富有特色的榕樹群樹冠造型。 這樣一個單層建築所佔據的面積為35英畝（141000平方米），它坐落在高雄市中心佔地面積為116英畝（470000平方米）的亞熱帶公園裡，並成為了世界上最大的單屋簷建築表演藝術中心，這也是臺灣這一代人的一項最重要的文化投資。 它共擁有五個藝術表演空間：一個可容納2,236個座位的歌劇院，一個可容納1,981個座位的音樂廳，一個可容納1,210個座位的劇場，一個可容納434個座位的獨奏廳和一個連接建築和公園的室外劇院。高雄市位於臺灣南部，擁有約300萬的人口，這裡曾經是一個重要的國際港口，如今已成為一個現代化、多元化的城市，擁有豐富的文化底蘊。 這座新建的國立高雄藝術中心位於一個原軍事訓練基地的舊址上，這象徵著高雄市和臺灣的發展前景——同時也肩負著通過藝術和文化將本地和國際化人才聯繫在一起。該專案藝術總監簡文斌（Chien Wen-Pin）說道，「來到衛武營旅遊的海內外遊客看到的是我們對戲劇、舞蹈、景觀和音樂的熱愛，這種熱情在臺灣隨處可見。 我們的觀眾也都是熱情而又知識淵博的。 我將繼續與國內外的藝術家一起工作，去探索能夠反映當代最佳實踐的新思路。 在衛武營我們擁有的這座偉大的建築，將會擁有更多的創新機會 —— 我們要大膽創新，嘗試不同的東西。 ”在2018年10月開幕的國立高雄藝術中心將完成臺灣地區表演藝術中心的傘式組織結構，其中包括臺北國家大劇院、音樂廳和國立台中大劇院。 這三家旗艦機構將成為臺灣國際知名的當代舞蹈和戲劇公司、中西合奏樂團、手偶劇團、奧斯卡和金獅獎獲獎製片人、圖書提名作家、華語流行歌星等人的大本營。

- Data from Mecanoo

National Kaohsiung Centre for the Arts

Kaohsiung, Taiwan

Discipline

Architecture, Interior, Landscape

Typology

Theatre/Concert Hall

Size: 141,000 m2
Status: Completed
Project Design: 2007 - 2011
Project Realisation: 2010 - 2018
Address: No. 1號, Sanduo 1st Road, Fengshan District, Kaohsiung City, Taiwan 830
Client: Ministry of Culture (MoC).

Design Team: Mecanoo (Lead Architect) and Archasia Design Group (Executive Architect).

Programme: Theatre complex of 141,000 m2 in the Wei-Wu-Ying Metropolitan Park with a total capacity of 5,861 seats (Concert Hall 1,981 seats, Opera House 2,236 seats, Playhouse 1,210 seats, Recital Hall 434 seats), exhibition space of 800 m2, 1,000 m2 of rehearsal/education halls for music and dance, two conference halls with 100 and 200 seats and stage building workshops, lobbies and restaurants, the grand covered public space Banyan Plaza, and 7-ha landscape design.

Awards: 1st prize Cityscape Architectural Award, Dubai 2008; 3rd prize International Design Award, IDA Los Angeles 2009; Chicago Athenaeum International Architecture Award 2009; Digital prize at Idea-Tops Awards 2017; Silver in the New York Design Awards 2018, Architecture - Public & Institutional; SDA Design Award 2018, Best Architecture and Spatial Planning; BUILD Infrastructure Awards, Best Civic Arts & Culture Venue Design, 2019; THE PLAN Award, Honorable Mention in the Culture category, 2019; Dutch Creativity Awards, Architecture Award, 2019; Architizer A+Awards, Special Honoree Award, Popular Choice Winner and Jury Winner in the Cultural, Hall/Theater category, 2019; DFA Design for Asia Awards, Silver Award, 2019; Fast Company’s Innovation by Design Awards, category Spaces, Place, and Cities, Honorable Mention, 2019; MIPIM Asia, Silver Award, Best Infrastructure Development and Special Jury Award, 2019; Taiwan Architecture Award, Honorable mention, 2019; International Architecture Awards, Museums and Cultural Buildings category, 2020; IES Illumination Awards 2020, Award of Merit.

Photography: Christian Richters, Iwan Baan, Shawn Liu Studio, Andrés Gallardo Albajar, Ethan Lee, Sytze Boonstra, National Kaohsiung Center for the Arts (Weiwuying), Ever Studio.

Project description

The National Kaohsiung Centre for the Arts symbolises the transformation of Kaohsiung, once a major international harbour, into a modern, diverse city with a rich cultural climate.

Mecanoo designed the Kaohsiung Centre for the Arts, located on a former military terrain, as an integral part of the adjacent subtropical park to have a positive social impact on the residents of Kaohsiung whose population counts almost 3 million.

Inspired by the local Banyan trees with their iconic crowns, the vast, undulating structure is composed of a skin and roof, and connects an extensive range of functions. The curved steel structure was built in cooperation between a local and a Dutch shipbuilder. Underneath this roof is Banyan Plaza, a generous, sheltered public space.

Residents can wander through here day and night, practice Tai Chi or stage street performances along walkways and in informal spaces. An open-air theatre nestles on the roof where the structure curves to the ground, with the surrounding park forming the stage.

Designed with the subtropical climate in mind, the open structure allows the wind to blow freely through Banyan Plaza. The seamless flow between interior and exterior creates opportunities for crossovers between formal and informal performances.

Different theatres such as the 1981-seat Concert Hall and 2236-seat Opera House are located in the five cores or ‘legs’ of the building where the structure meets the ground. The cores connect with one another via foyers in the roof and an underground service floor which houses the backstage area of each theatre.

- Data from 台灣建築學會

從數位設計到現場施做 衛武營藝術文化中心

文／侯慶謀、林宛蓁

衛武營區位於舊高雄縣、市交界，自日治後期這塊基地即作 為軍事基地使用，之後做為國防部新兵訓練營區。2007年3月衛武 營藝術文化中心國際競圖揭曉，由來自荷蘭的麥肯諾建築師事務 所(Mecanoo Architecten)，自全球41件參賽作品中脫穎而出，贏得首 獎。基地面積約10公頃，樓地板面積約14萬平方公尺，總工程預算 約81億。

榕樹廣場

衛武營藝術中心的有機設計源於基地內生長了幾十年、獨具特 色的繁茂榕樹林，建築師法蘭馨荷本(Francine Houben)第一次來台灣 時，觀察到樹蔭下的空間，人們三兩聚會，微風徐徐吹過，這正是 熱帶地區公共空間的最佳原型。在競圖方案中， Mecanoo創造出一 個土地與建築之間的強烈連結，樹林創造出遮蔽性的半戶外空間， 反應在建築物的曲面與穿透性的空間設計上。我們轉化基地上榕樹 的空間結構，而此連續曲面的材料與工法，則是結合高雄本地造船 工業，打造台灣人民的榕樹廣場—台灣的新文化地標。

數位輔助設計

設計發展過程中，考量全案室內機能的複雜 度、各種機能需求、工期、預算、台灣可行的工 法與施工能力等，建築設計的第一步是將外殼與 內裝做出幾何上的分別，將複雜的自由曲面限制 在外殼，其他部份全都以一般放樣與製造技術可 以處理的幾何形來設計。 Mecanoo設計團隊內也分成Digital Project與 Rhino兩個3D模型操作組，分別處理外殼與內 裝的數位建構；瀚亞建築師事務所、劇場顧問 theateradvies、聲學顧問Xu Acoustique都使用Rhino與 所有設計團隊溝通；結構顧問超偉工程則以Rhino 結合SEB、ETABS與建築模型套繪；鋼表皮顧問 VUYK使用Rhino與NUPAS、ANSYS設計；燈光顧 問月河使用Rhino與ECOTECK、AGI、DIAlux；其 他顧問得要透過3D顧問利道協助建模與檢討。

高雄造船廠打造鋼表皮

榕樹廣場自由曲面表皮最大的困難點，在於 工法材料的選擇。我們研究了多種可能方案，包 括鋼板、鋁板、混凝土、合成薄膜、Stucco等，考 量的重點包括曲面精度的控制、強度、耐久性、 台灣產業可行性、建築與消防法規需求、預算及 工期。我們藉由2008米蘭家具展的一個機會，製作榕樹廣場1：5的縮尺模型，測試材料的一種可 能性。最後決定以“樹根”投影線為分界，地板 以RC加上PU施作，其他的部分以造船鋼板製造。 榕樹廣場鋼板表皮面板總組立面積約2萬3千 平方公尺於現場組立之板片重量約為1.2至2.4噸之 間，均為3D不規則形狀。在建築與造船異業合作 的案例中，本案是目前全世界最大規模的一個， 也是目前唯一需要同時處理風力與地震力等結構行為的“陸上造船”。 相較於建築產業大部分的施工圖還是以切面 投影方式呈現，造船業的3D模型、分析與製造資 訊都整合在同一軟釐米，到現場安裝時，僅需將 一塊塊“積木”定位並焊接，精度控制同樣達到 釐米，遠較土木工程或一般建築工程的精度高， 並且利用工廠預製的條件，大幅提高精度與品 質，大量縮短工期。此外，製造的方式採用造船 的“冷彎”技術，所有版片利用電腦分析，直接 輸出成為版片機械加工的數據，減少人力，並降 低人為錯誤的機率，更免除“熱彎”形成的內應 力與不準確性。 Mecanoo設計的自由曲面外皮，是由荷蘭的專 業造船顧問VUYK，提供造船業慣用的規範及設 計圖說，再由美國帷幕牆顧問CDC轉為台灣的規 範與圖說，接著由建國工程承攬，慶富造船實際 製作，結構計算由崑山大學的團隊協助。整個過 程，挑戰了大家習以為常的作業方式與體制，在 全世界具有指標性意義。

屋頂下一公尺的學問

屋面的自由曲面除了外型精度上的控制以及 預算工期的考量外，聲學上需要達到33dB的隔音效能，且結構上對於屋面重量也有相當限制，最 後我們選擇預算內的系統與“Standing Seam”鋁 板 #7。這個工項的挑戰來自於自由曲面上，要維 持所有Standing Seam紋路都能在投影平面上平行， 使得超過50%板片都需要客製化。而施工上，屋面 的施作必須要能夠掌控鋼構到完成面一公尺內的 十二道工序，才能將精度達釐米的3D彎板順利安 裝完成。完整工序經由一比一實體樣品的驗證 ， 經歷過拆除重做一次的學習。 因此，德國的屋頂廠商BEMO必須派一位3D 測繪工程師到台灣數次，一一確認現場已完成組 裝的構件高程，依照現場的真實資訊，將各階段 的高程數據回饋給其他合作單位，並將一台3D版 片成形機運送到台灣現場加工，版片分析與製造 過程也全部數位化。由於BEMO只有一台機器可以 滿足本案的複雜度，衛武營的工期必須與BEMO在 全球其他的項目協調妥當，才能確保機具能夠準 時出現在工地現場。

BIM的精神與效益？

衛武營藝術文化中心包含一個戲劇院2260 席、音樂廳2000席、中劇院1250席與演奏廳470 席，以及大量配套空間。劇場的設定經過業主(當 時文建會)兩次重大變更，數十次設計評審委員會 討論，最後的劇場設計需求達到非常高的標準， 建築師與劇場顧問們，將複雜的劇場機能與高規 格的劇場設備，毫無妥協地整合進有機榕樹量體 中，更困難的是：整合進複雜的台灣法規框架 內。另外，聲學的設計需求分為兩部分，一個是 廳內的殘響時間，另一個是背景噪音的控制。音 樂廳、演奏廳與樂團排練室的背景噪音值都必須 達到NC15的要求，相較台北兩廳院的NC20，衛武營的要求嚴格得多。為了達到合約要求，所有的 空間規劃、構造工法、機電設備與帷幕牆設計等 等都必須配合。全案的設計考量重點多如牛毛， 牽一髮動全身，在施工階段與營運階段，都必須 毫無折扣的執行，但要如何達成？ 在施工階段，總共分為七標，包含一標主體 結構工程(建國工程/春源鋼鐵)、二標建築裝修機 電空調工程(建國工程/中興電工/世益空調)、三標 劇場特殊設備工程(亞翔工程/榮工/Waagner-Biro/ 宜盛科技) 、四標管風琴採購(琴峰企業/Klais/豐澤營造)、五標景觀工程(永青營造)、六標捷運地下 連通道(高雄捷運局代辦)、七標營運辦公區裝修 工程(截稿前尚未發包)。各標預算中編列了3D整 合費用，讓廠商各自依其產業習慣繪製3D模型， 再到共同平台進行套繪，完工時交給營運單位使 用。實際執行過程中，有些公司藉此機會成立BIM 部門，將公司帶向另一個層次，雖然有的半途而 廢，有的成立了新公司專門接相關業務，也有些 公司發包給專門建3D的公司處理。 過程中所有廠商共同的問題，除了因業主、PCM要求“快捷工法(fast track)”造成分包期程、 設計、介面整合的困難，最多狀況都是營造廠下 包發包作業困難、人員流動頻繁造成資訊斷層、 廠商間的溝通不順暢、各單位不理解造成不支持 等狀況，使得3D模型建製的功效受限，大量的工 序與衝突檢討還是持續在現場上演，低價搶標的 營造廠只會越來越辛苦，將來業主、PCM、建築 師都要面對無數可以避免的“爭議”。 舉個例子，劇場與音樂廳這樣的機能並不是 工程中經常出現的，加上高規格需求，讓許多設 備與工法，對於承包廠商甚至是設備的進口代理 商都是在台灣的第一次。演奏廳與排練室的盒中 盒技術，必須靠鋼彈簧將內層RC樓板撐起來，不 能與外盒有任何接觸，現場發生過協力廠商在不 清楚工法的狀況下，將管線埋在內層RC樓板穿出 排練室，造成地板無法“浮動”，還好這個錯誤 有方法補救。劇場設備有許多動態移動的元件， 但是整合檢討結束後，依然可以發現與他標的衝 突未呈現，原因是建模尺寸有誤，還好設計單位 主動了解，並到現場一一說明給各標負責工程 師，即時處理掉一個“未爆彈”。這些資訊在營 造廠的3D整合檢討過程中，應有機會將種種資訊 紀錄下來，並要求所有夥伴確時執行。 然而，BIM的“精神”仍在本案的一些部分發 光發亮，例如：讓衛武營能夠昂然站立、展現其 優雅曲線的主體鋼結構，21,500公噸的總用鋼量， 足以興建3座艾菲爾鐵塔。這個龐大的工項，春 源鋼鐵花了近半年的時間，製作3D模型、實體模 型、2D圖面來研究如何分區、動線如何安排、吊 裝順序等，並仔細研究了各式結構單元類型與吊 裝形式，包含各種高空作業的可能方案。皆下來 分析所有構件類型，並依運輸計畫來裁切鋼構單 元尺寸。事前詳盡的準備，讓工程能順利接續進 行。 BIM精神在本案另一個工項-榕樹廣場鋼表皮 充分落實，參與本案的造船產業，為我們做了一 個不錯的示範，從設計到施作都可一窺其產業鏈 上下之續接性與成熟度。值得注意的是，主體鋼 結構與榕樹廣場鋼表皮這兩個範例，都不是用註 冊BIM的Autodesk Revit軟體來達成。

跨領域、跨階段的展望

3D技術的發展讓工程多一項工具協助工程 進展，但其實工程順利與否，還是在使用工具的 人。各專業廠商龐雜的資訊是否都能無遺漏的輸 入到模型中？衝突檢討怎麼作？這些設計與工程 資訊是否到達每個工人以及營運團隊的手中? 2013年的美國建築師學會(AIA)年度白皮書 中，揭櫫當代建築師首重跨領域的理解與合作， 無論是建築產業中各個專業的合作、學術與業界 之間的合作、甚至是跨產業的合作。在面對今日 以及未來的挑戰，建築師無法僅僅顧守本位。建 築營造產業演變至今，它對於建築師的要求，除 了靈光一現的創意外，它需要建築師與其他專業 攜手向前，無論是衝突、協商或交流，建築透過 與產業內各專業的連結而更加完善，而透過與學 術，甚至與不同產業所激盪出的火花，更將為建 築帶來新象與突破。

- Data from Archello

Kaohsiung is an informal, lively city of almost three million inhabitants. Not only is it the second largest city in Taiwan and one of the world’s largest sea ports, but also host to a dramatic sub-tropical climate of typhoons, high temperatures, heavy rainfall and regular earthquakes. The new 140,000 square-metre performance complex must cope with all of these extremes.

Barking dogs and banyan trees

The location for this building is a former sixty-five hectare military compound in the centre of the metropolitan area of Kaohsiung – a place which is intended to become a sort of ‘Central Park’ for the city. When we first visit the site in 2006 it is still a fenced-off zone of military barracks occupied by a lot of wild, barking dogs. It’s also dense with banyan trees, their twisting trunks and branches having gradually interlocked over decades. Their aerial shoots grow down into the soil forming additional trunks, spreading over an astonishingly wide area, and the crowns of these trunks become one, like a huge umbrella. According to local legend, Alexander the Great once sheltered his entire army beneath the crown of a single tree.

The form of the banyan reflects the local climate. Its wide crown, providing shelter against the sun and rain, is a perfect expression of Kaohsiung’s humid atmosphere. In the sub-tropics the sun sets at around six or seven o’clock and, just as it disappears beneath the horizon, the city comes to life. During these twilight hours as the temperature cools people stroll the streets, shop for their groceries, and eat outside. Under the canopies, as well as under the trees, informal performances occur: people dance, play music, perform plays, or practice Tai Chi. What we see, hear, feel, taste and smell on this first trip to Taiwan inspires us to somehow harness this unique urban atmosphere.

A continuation of the park

We wonder whether the banyan tree could help us to create a combination of formal and informal performance spaces. In this way the building would be full of life around the clock; a natural continuation of the park. We know that good public buildings need good public space, a mantra which is central to our approach to civic architecture: think of the Library of Birmingham’s sunken amphitheatre, or La Llotja de Lleida’s urban stage beneath a cantilever, using the stairs of an adjacent building as audience seating. The formal qualities of these three buildings are very different but they have one thing in common: they are a perfect match with their respective locations, cultures and climates.

The open, protective shape of the banyan tree becomes a springboard for the design. Its expansive sheltered crown becomes the Banyan Plaza, a partially enclosed public space where the cooling wind can freely flow. Between the four formal performance halls, which form the ‘trunks’ that support the undulating roof, a topography rising from ground level to plus five metres becomes part of the park’s landscape. Where the roof touches the earth the building becomes an amphitheatre, open towards the surrounding parkland which, in turn, becomes both its stage and its set.

Cargo ship

Each performance venue and foyer has its own unique atmosphere and together create one fluidly connected space. The walls and continuous ceiling surface of the Banyan Plaza is a neutral white, allowing for artificial light and projections to alter the mood. With twelve chandeliers suspended against the billowing form of the plaza, the building can really dress up for the night! But a question remains: how do you materialise a space the size of a cargo ship, almost 160 metres wide and 225 metres long? Should it be clad in stucco, tiles, aluminium, or steel?

A Piece of Banyan

In 2008 we are invited to design a pavilion for the Salone del Mobile in Milan. We decide to build ‘A Piece of Banyan’ – a 1:4 scale model of a sliced fragment, twenty metres long and three and a half metres wide, to research and test the proposed ceramic tile cladding for the Banyan Plaza. We affix tiles to the model reminiscent of the craquelé of Chinese porcelain and we also test the ambient lighting. The changing colours and atmospheres are spectacular; the tile cladding on the curved facades, on the other hand, is less of a success. For one, it’s raining heavily in Milan and is impossible to glue the tiles to the ceiling.

We continue our search for the building’s skin and the idea arises to use the finely tuned, locally accessible techniques of Kaohsiung’s ship building industry. We convince everyone that the Banyan Plaza should have the detailing of a cargo ship, and not of a luxury yacht. There should be hoist points, for example, so that lights, banners and fabric can be hung. We don’t want to aim for a perfectly smooth façade – we want to employ the ship builders to do what they do best: create a durable, long-lasting skin on a scale that some contractors would be intimidated by. We ultimately want the building to look like a ship on the land.

The four halls

Each of the four performance halls has its own acoustic challenge, and so we collaborate with acoustician Albert Xu. He constructs a 1:10 scale model to test the performance of the most complicated venue: the 2000-seat Concert Hall. Despite its size we want the theatre to have an intimate feel. Wouldn’t it be great if every member of the audience could see the hands of the pianist? We choose the shape of a stepped vineyard with a stage at its centre, so that terraces at different floor heights encircle the podium. With seating on all sides of the stage, the audience is in close proximity to the performance itself. The public enters the oak-lined from the ground floor and spiraling ramps lead them to their golden seats.

The 2260-seat Lyric Theatre (also known as the Opera House) is arranged in the form of a horseshoe with three circled balconies. The seating is upholstered in a mixture of red and purple fabrics with a pattern of Taiwanese flowers, contrasting with the darker walls. This theatre is not only suitable for Western opera, with an orchestra of over seventy musicians, but can also be acoustically adapted to accommodate Chinese opera by manoeuvring a suspended acoustic canopy.

The Playhouse, with its 1,243 seats in Mecanoo Blue, is designed to host a variety of drama and dance performances. Flexibility is the core element in the design of this multifunctional space. The layout is adaptable and can be changed from a typical proscenium stage with the audience in front, to a thrust stage with the audience on three sides.

The 470-seat Recital Hall has the most intimate atmosphere of the four. With its asymmetrical composition and seating across two levels, it is designed for chamber music and recital performances. The seats have the same golden fabric as the Concert Hall and oak lines the walls. The upper part of this hall is enclosed by a circle of sound absorbing curtains, allowing for the reverberation time within the space to be tuned to specific types of performance.

Consultants

Structural engineer: Supertech, Taiwan

Mechanical engineer: Yuan Tai, Taiwan

Electrical engineer: Heng Kai, Taiwan

Acoustic consultant: Xu-Acoustique, France

Theatre consultant: Theateradvies, The Netherlands; Yi Tai, Taiwan

Lighting consultant: CMA lighting, Taiwan

Fire safety: Ju Jiang, Taiwan

Organ consultant: Oliver Latry, France

Roof and façade consultant: CDC, USA

3D consultant: Lead Dao, Taiwan

Traffic consultant: SU International, Taiwan

- Data from THE PLAN

Part landscape, part architecture, the 141.000m2, National Kaohsiung Center for the Arts tucks a collection of 4 performance halls beneath a 35-acre artificial terrain that rolls like hills. Floating at one end of a park that had been a military base, the complex—the largest performing-arts center under one roof—seems to ooze from below its immense lid, blurring the boundaries between indoors and out, solid and void. Mecanoo created an undulating middle realm between roof and ground that provides access to four indoor auditoria while remaining open to breezes and views of the park on all sides - the Banyan Plaza.

Innovations:

Inspired by the large banyan trees that grow on the site and provide welcome shade, with their long, arching branches, we enveloped this interstitial space in a curving steel canopy that slides down to wrap around the elliptically shaped theaters. Called Banyan Plaza, it provides a cool retreat from the subtropical sun and frequent rains—a place where anyone can come to do tai chi in the morning, jog in the afternoon, or watch films projected on its underside in the evening. It defines the cultural center’s public space. This fusion is also evident in the outdoor amphitheater - where the roof, which incorporated the tiered seating, touches the ground to allow the green space to become the stage. Given the complex formal language, we created a parametric building model with the "Digital Project" program developed by Frank Gehry's office in order to better cope with the planning changes.

Environmental qualities & sustainability:

A built-up roof resting on a steel frame buffers the performance halls underneath from the sound of rain and outdoor noise while etching an eye-catching profile against the horizon and acting as the projects’ fifth facade. As for the materialization of the skin, we decided to use steel plates rather than poured-in-place concrete to envelop Banyan Plaza’s curvaceous spaces and enclose the performance halls. Large steel springs behind the steel skin act as dampers and allow the building envelope to move independently of the structure during seismic events and storms. Using steel panels in the high-traffic public plaza also reduced maintenance issues. A mechanical plant for the entire complex is placed underground to reduce noise. For the same reason, engineers specified a displacement ventilation system, with ducts in the floors supplying conditioned air at low velocities to cool the space around theater-goers. Large openings into the roof above bring daylight.

Integration in its environment & benefits to the community:

With 2.7 million inhabitants, Kaohsiung is the largest port city in Taiwan and one of the largest in the world. The cultural centre in the 65-hectare WeiWuYing Metropolitan Park seats 6000 people in its opera house, playhouse, concert hall and recital hall. Such a programme need a lot of space, but it's extraordinary that such a building - 225m long, 160m wide and up to 36m high - fits so naturally into both the surrounding city and park. The complex reflects Kaohsiung’s transformation over the past 15 years from a harbor city to a diverse metropolis, where parks and cultural facilities attract well-educated Taiwanese and offer tourists a reason to visit. During the 12 years, it took to design and build Weiwuying, the local and national governments have connected the city to the capital, Taipei, and to Taichung, via high-speed rail, and opened a subway system for Kaohsiung.


The factors that distinguish Mecanoo's design is its response to the location and its open invitation to anyone and everyone to use the building. Our futuristic re-interpretation of the banyan tree is central to the form concept, the local shipbuilding industry was brought into the project, and the street theatre and colour of the local culture inform the offer of the Banyan Plaza. The National Kaohsiung Centre for the Arts opens up public realm that draws the city's people, not just performance tickets -holders, into the structure itself. And as for performance, the auditoria are world-class for formal works, but anyone can enjoy informal acts in the plaza, or popular concerts in the park. This is more than a grand and elegant building for the elite - it is for the people.

Programme: Theatre complex in the Wei-Wu-Ying Metropolitan Park with a total capacity of 5861 seats: Concert Hall 1981 seats, Opera House 2236 seats, Playhouse 1210 seats, Recital Hall 434 seats, exhibition space of 800 m2, rehearsal/education halls for music and dance, 2 congress halls with 100 and 200 chairs and stage building workshops. Including 7ha landscape.

- Data from Dezeen

Mecanoo's 141,000-square-metre National Kaohsiung Centre for the Arts, described as the "world's largest performing arts centre under one roof", is now open in Taiwan.

Enclosed beneath a bulging 34,843-square-metre roof, the building is known as Weiwuying, after the neighbouring park.

It contains four new indoor performance venues: a 2,236-seat opera house is located at its centre, with a 1,981-seat concert hall, a 1,210-seat playhouse and a 434-seat recital hall arranged around it.

There is also a fifth performance venue – the building's roof dips to touch the ground, forming an outdoor theatre facing the park.

The building's flowing roof is defined by the scale and arrangement of the four auditoriums contained within.

"We wanted to develop a new typology, one that takes advantage of the local climate, culture and size of the programme, explained Nuno Gonçalves Fontarra, associate partner at Mecanoo.

"The typology of the building is a hybrid between a single block and a complex of several individual venues," he told Dezeen.

The art centre's auditoriums are all accessed from a covered foyer, named the Banyan Plaza, which runs throughout the building around the venues, creating a publicly accessible area for people to gather.

"All venues are connected by a public square like it would happen in a complex of several buildings," explained Fontarra.

"The difference in our case is that this space is covered and unified with a continuous language. By generating this urban public void – a cave-like space – it serves as a protected retreat from the busy urban environment, in the same way people meet and greet in the shade of the banyan trees."

Banyan trees, which are commonly found in the region, were the starting point for the creation of the building's overall form.

"A cultural building of enormous scope and prestige, Weiwuying is based on the local banyan tree, and made by the local shipbuilding industry," said Francine Houben, founding partner and creative director at Mecanoo.

"The banyan is a very large and striking tropical tree. In Kaohsiung, everyday events, encounters and gatherings take place literally beneath the banyan trees," she continued.

"As I spent time in Kaohsiung, and studied the shape of these magnificent trees, they revealed a formal language that gave us the overall theme for our design."

"The plaza works like a banyan tree, gathering and sheltering people while allowing easy movement in and out," said Houben.

"With its walkways and informal spaces, it allows people to do all the things they do beneath a banyan tree – even create impromptu performances."

Above this covered plaza are the building's other facilities, including rehearsal and event spaces, restaurants and an art gallery. The technical areas, which serve the theatres, are located below the foyer space.

According to Mecanoo, each of the four auditoriums has its own identity, layout, acoustics and ambience. The concert hall is designed with a vineyard style, the recital hall is more intimate and the opera house is more grand.

"All the venues have very different functions and that is materialised in a totally different seating and room layout," said Fontarra.

"The concert hall has a vineyard layout, the opera house is designed in the historically famous horseshoe shape, the playhouse is fully adaptable to a range of layouts, therefore the arrangement is of a thrust theatre."

The seats in the concert hall and recital hall are a champagne colour, while the opera venue has red seats and the theatre's seating is "Mecanoo blue".

National Kaohsiung Centre for the Arts is built on the site of a former military base. It forms part of a newly created 47-hectare park, alongside the existing Weiwuying Metropolitan Park in the east of the city.

Mecanoo has also recently completed a vast law court in Córdoba, Spain, and transformed a gasworks in the city of Hilversum, the Netherlands, into a residential neighbourhood featuring cylindrical apartment buildings.

Photography is by Iwan Baan.

Project credits:

Client: Preparatory Office of The Wei-Wu-Ying Center for the Arts of the Ministry of Culture
Architect: Mecanoo architecten
Local architect: Archasia Design Group
Structural engineer: Supertech
Mechanical engineer: Yuan Tai
Electrical engineer: Heng Kai
Acoustic consultant: Xu-Acoustique
Theatre system: Waagner-Biro
Theatre consultant: Theateradvies; Yi Tai
Lighting consultant: CMA lighting
Fire safety consultant: Ju Jiang
Organ consultant: Olivier Latry
Roof and facade consultant: CWI
3D consultant: Lead Dao
Traffic consultant: Su International
Contractor: Chien Kuo Construction
Steel skin: Centraal Staal
Organ builder: Klais