- Scale

Grand Theatre: ~1,500 seats

Globe Theatre: ~800 seats

Blue Box: ~600 seats (flexible)

Total GFA: ~57,000 m² · 國家級

- Location

Continent: Asia

Country: Taiwan (R.O.C.)

City: Taipei · Shilin District

City population: ~2.6 million

Program — 綜合性劇場 (Multi-venue performing arts centre)

Grand Theatre (大劇院): proscenium + thrust, ~1,500 seats, opera/dance/drama

Globe Theatre (球劇場): rotating spherical volume, ~800 seats, immersive/in-the-round

Blue Box (藍盒子): flexible black box, ~600 seats, experimental / flat-floor configurations

Underground parking · night-market plaza at grade · administration offices (fly towers above)

- Form

Massing: rectangular "super-stage" cube raised ~15 m above grade; three theatre volumes

plug into all four sides — one sphere (Globe) protrudes as a full cylinder; Grand Theatre

angles off the SW corner; Blue Box anchors the south. The cube reads as a monolith.

Envelope: perforated aluminium cladding on the raised box; exposed concrete base; Globe

Theatre clad in ETFE-backed aluminium panels forming a gridded sphere. Skin is flush,

flat, and industrial — contrasts with the organic sphere and the night-market context.

- Site / Context

Adjacent to Shilin Night Market —

dense, informal, commercial urban

fabric. Ground floor fully open:

plaza flows under the building,

linking to MRT Jiantan station.

- Composition

Spatial order: shared "super-stage"

core serves all three theatres.

Circulation: public loop (+0/+4.5/+6.5 m)

wraps exterior; backstage vertical cores

separate from audience routes.

Accessibility + Circulation detail

Ground plaza → escalators/lifts → lobby at +0 → public loop at +4.5 and +6.5 m navigates

around fly towers and globe exterior → foyers of each theatre. Parking via B1–B2.

- Participants

Architect: OMA (Rem Koolhaas + David Gianotten) · Local architect: Huang Da Architects

Client: Taipei City Government (Department of Cultural Affairs)

Operator: Taipei Performing Arts Center · Structural: Arup

{

"project": "台北表演藝術中心 Taipei Performing Arts Center (TPAC)",

"scale": {

"seats": {

"grand_theatre": 1500,

"globe_theatre": 800,

"blue_box": 600,

"total": 2900

},

"gross_floor_area_m2": 57000,

"theatre_scale": "國家級 / National-level"

},

"location": {

"continent": "Asia",

"country": "Taiwan (R.O.C.)",

"city": "Taipei",

"district": "Shilin",

"city_population": "approx. 2,600,000"

},

"program": {

"theatre_type": "綜合性劇場 / Multi-venue performing arts centre",

"venues": [

{

"name": "Grand Theatre 大劇院",

"stage_type": "Proscenium + thrust stage",

"capacity": 1500,

"use": "Opera, dance, large-scale drama"

},

{

"name": "Globe Theatre 球劇場",

"stage_type": "In-the-round / rotating spherical auditorium",

"capacity": 800,

"use": "Immersive and experimental performance"

},

{

"name": "Blue Box 藍盒子",

"stage_type": "Flexible black box / flat-floor",

"capacity": 600,

"use": "Experimental theatre, rehearsal, site-specific"

}

],

"ancillary": [

"Underground parking B1–B2",

"Public plaza at grade",

"Administration and rehearsal offices",

"Shared super-stage backstage infrastructure"

]

},

"form": {

"massing": "Rectangular super-stage cube (approx. 60×60×35 m) elevated ~15 m above grade. Three theatre volumes plug into its four faces: Grand Theatre angles off the SW corner, Globe Theatre protrudes as a full sphere from the north, Blue Box anchors the south. The cube reads as a datum; theatres as insertions.",

"envelope": "Perforated aluminium cladding on the raised cube body. Globe Theatre wrapped in aluminium panels with ETFE backing, forming a structural gridded sphere. Base is exposed concrete. Roof carries green terraces and mechanical equipment.",

"skin": "Flat, industrial, flush surfaces throughout. Contrast between the machine-like cube and the organic globe geometry. No ornamental hierarchy — the building's form IS its ornamentation.",

"form_program_relationship": "Shared super-stage core determines cube dimensions; three distinct audience geometries (fan, sphere, flat) each extrude their own volume outward from the central box."

},

"site": {

"context": "Adjacent to Shilin Night Market — dense, informal, commercially active urban fabric. Site served by MRT Jiantan station (walking distance). High pedestrian flow, predominantly low-rise surroundings. Ground floor left fully open as public plaza to preserve night-market circulation logic."

},

"composition": {

"spatial_order": "Centralized super-stage cube as spatial and structural datum. Three theatres orbit it, each independently accessible yet all sharing backstage resources. Offices stacked above in a separate wing connected to the cube.",

"circulation": {

"public": "Grade-level plaza → escalators/lifts → lobby at +0.0 m → public loop at +4.5 m and +6.5 m wrapping around fly towers and Globe exterior → individual theatre foyers",

"backstage": "Dedicated vertical cores separate from public routes; loading docks at grade",

"parking": "B1–B2 underground levels accessed from perimeter"

},

"accessibility": "Ground floor fully open and column-free at key passages; lifts integrated into public loop; MRT connectivity reinforces pedestrian access"

},

"participants": {

"architect": {

"design": "OMA — Office for Metropolitan Architecture",

"principals_in_charge": ["Rem Koolhaas", "David Gianotten"],

"local_architect": "Huang Da Architects & Associates (Taiwan)"

},

"client": "Taipei City Government, Department of Cultural Affairs",

"operator": "Taipei Performing Arts Center (TPAC)",

"consultants": {

"structural": "Arup",

"theatre": "Theatre Projects Consultants"

},

"user": "General public, performing arts companies (resident and touring), Taipei citizens"

}

}