- Data from www.levittbernstein.co.uk

Project Details:

A unique, ambitious, seven-sided theatre-in-the-round, designed within a grand Edwardian Cotton Exchange in Manchester.

Client: Royal Exchange Theatre Company

Construction Value: £18.5m

Completion: 1976 (first works) and 1998 (second works)

Location: Manchester

Awards:

- Manchester Society of Architects’ 160 Favourite Building – The People’s Choice, 2025: Winner

- Civic Trust Awards 2000: Commendation

- RIBA Awards 1977: Winner

- The Times RICS Conservation Awards 1977: Commendation

- Structural Steel Classics Awards 1906-86: Recipient

Where we started

We were initially invited to work on the Royal Exchange project in the 1970s by a nascent theatre company, led by Braham Murray, Michael Elliott, James Maxwell, Casper Wrede and Richard Negri. Together, they had a vision for an immersive theatre experience for both performers and the audience. All they needed was somewhere to put this into action.

A home was eventually found in the Royal Exchange, a vast, regal, three-domed Edwardian building in the centre of Manchester. Originally built to trade cotton at the height of the city’s industrial pomp, it had closed in 1968 and the Grade II listing ruled out many conventional uses. Having tested its viability with a season of productions, the theatre company negotiated a lease and asked us to develop their concept of a theatre-in-the-round into a permanent fixture. Our clients were a group of the company’s artistic directors, designers, actors and technicians, all passionate about their shared vision and the potential of this glorious space. They didn’t want their architects to have any preconceptions about what a theatre should look like or how it should function, and we were appointed on the basis that we had no arts experience whatsoever – the mark of a brave and imaginative client.

This project was a true collaboration and through a constant exchange of ideas, a building slowly emerged. From the start, we envisaged a lightweight structure boldly inhabiting the space, its modernity in stark contrast to the traditional splendour of the hall. The seven-sided construction spans between existing columns to avoid loading the delicate floor and, internally, an odd number of facets ensures that members of the audience do not directly face each other. The finished article was likened to the lunar module that had accomplished its epic journey just a few years earlier.

The atmosphere is breathtaking as one enters that huge foyer space of the old nineteenth century Royal Exchange to see, suspended from the four monumental columns supporting the central dome of the building, this lunar space module-like structure of steel and glass that is the theatre itself.

Richard Negri, RET Director

As testament to the original vision of creating an experience, the design cleverly brings the audience and performers closer together. No member of the audience is more than ten metres from the centre of the stage, and the dressing rooms, along with the café and front of houses areas, are located within the hall. This means that the general public and actors share space beyond performances. Equally, the technical workings of the theatre, usually hidden from the audience, are visible on the outside of the structure.

The original design was completed in 1976, but just 20 years later, the 1996 bomb caused devastating damage. We were invited back to undertake the necessary restoration work and given the opportunity to make some radical changes to the rest of the space.

A new entrance was inserted into the fabric of the old building with a lift up to a glass walkway, making the space-age module more visible to passers-by. A new 120 seater studio space was also introduced into the old hall, enabling the theatre to develop ideas, test out new productions, and work with different audiences.

- Data from architectsjournal

This seven-sided theatre in the round, suspended within Manchester's three-domed Edwardian Cotton Exchange and designed by Levitt Bernstein in 1976 is one of more than 3,200 projects that you can find in the AJ Buildings Library

The design of the auditorium aimed to give a lightness and character opposite to that of Manchester’s Edwardian Cotton Exchange in which is it housed. It was therefore designed as a suspended seven-sided theatre in the round.

All other areas of the theatre, including the restaurant, rehearsal rooms, offices, dressing rooms and service areas, were incorporated within the original cotton exchange building.

The structure is carried on the four brick piers which also hold the vast dome of the original building. A 30 x 21m rectangle sits between the piers and dictates the size of the theatre. Four trusses form the principal walls of the module with additional trusses forming the roof. Steel mesh panels, toughened glass and steel decking enclose the structure.

The theatre provides a separate acoustic entity with seating for 700 people spread between cantilevered galleries and the existing floor plate.

Project data

Begun 1975
Tender date 1974
Completed 1976
Total cost £775,000
Architect Levitt Bernstein
Client Royal Exchange Theatre Company
Structural engineer Ove Arup & Partners
Quantity surveyor Mahon & Scears
Services engineer Max Fordham and Partners
Quantity surveyor Monk and Dunstone
Theatre services Theatre Projects
Acoustics D. K. Jones
Sound Ian Gibson

Royal Exchange Theatre Patrick Morreau

IntroductionThe Manchester Royal Exchange, within which the Royal Exchange Theatre is now housed, was built during the First World War and is itself an extension of the Cotton Exchange built between 1866 and 1874, which in turn replaced two earlier exchanges. An air raid in 1941 destroyed the original part of the building but left the present hall largely undamaged. Although the Manchester Cotton Exchange at one time numbered 12,000 members, trading ceased on 31 December 1968 and the Great Hall stood empty.

In 1972 the '69 Theatre Company, at that time based at Manchester University Theatre, obtained a 25-year lease from the Prudential Assurance Company, owners of the Royal Exchange, and set about the formidable task of creating, within the first and mezzanine floors of the Exchange, a theatre to seat 700, complete with workshops, rehearsal rooms, offices, bars and a restaurant, along with cloakrooms, toilets and ticket offices.

Richard Negri, an artistic director of the company, was appointed to be responsible for the conceptual design of the theatre itself, and Levitt Bernstein Associates as architects for the entire project. Ove Arup and Partners were appointed structural engineers, and subsequently their brief was extended to include fire safety engineering.

The initial conception of the theatre, a development of the ideas used by the '69 Theatre Company in their earlier productions, was of a free-standing concentric auditorium within the Great Hall, creating a close and vibrant relationship between performers and audience. This intimacy was to be achieved by seating the audience around the performing area, at the level of the stage and on galleries above, so that none should be more than $10\text{ m}$ from the centre of the stage. Actors were to be able to enter from several points around the perimeter, scenery to be flown from above the performing area, which itself was to be adaptable to different sizes and configurations, and the traditional 'upstage', 'downstage' and proscenium dispensed with.

This concept, which has remained little altered by the vicissitudes of design development and budgetary rigour, evolved into an open stage theatre, seven-sided in plan, with stage and seating for 450 at the level of the Exchange floor, and two galleries above, each of which seats a further 150 people. Although the Exchange building itself provides protection from the wind and rain, a further enclosure is needed to provide, within the auditorium, a controlled thermal and acoustical environment; thus the theatre is clad with toughened glass and roofed with metal decking.

StructureThe structural design started, as it commonly does, with a site investigation, with the difference that in this case the site to be investigated was the structure of the existing building, and the data sought was not the bearing capacity of the soils but the strength of the existing floors, columns and foundations. In this investigation we were immeasurably helped by Mr T. Pevitt of Bradshaw Gass and Hope, the Bolton firm who had been the architects and engineers of the original building and who have, for one reason or another, been involved with the Exchange almost continuously for 60 years. They were able to provide us with copies of all the original drawings and to inform us where and how the actual construction was changed from that shown on the drawings because of materials shortages during the closing years of World War One.

This investigation revealed that the existing floor construction was not capable of supporting the two galleries or the roof of the auditorium but that limited additional load could be carried by the brick piers of the Exchange. The architectural and engineering problem, then, was to arrive at a structure which would transfer the loads of the upper levels of the auditorium to these piers without obstructing free circulation on the Exchange floor, without sacrificing the sense of the Great Hall as a vast, single volume, and without generating more load that the piers could support. The limitations set by this problem of support became, in the words of the architects, the most influential factors in determining the final form of the auditorium and assisted rather than hindered the development of the design.

The auditorium is placed directly under the central and largest of the three glazed domes which light the Great Hall, and it is the piers that carry this dome which are used to support the roof and galleries of the auditorium. These piers, of solid brick construction faced with scagliola, are approximately $3\text{ m}$