Oslo Opera House

- Data from archdaily

Architects: Snøhetta

Area: 38500 m²

Year: 2007

Manufacturers:  RMIG, Ruukki, Frapont, Bosvik AS, Buchele, Campolonghi, ES-System, Falcon Lifts, Glatz, Inotec, Ludvig Svensson, Naturstein Montering AS, Nicopan, SML Lighting AS, Stretch Ceilings, Vestre

Acoustics: Brekke & Strand Akustikk, Arup Acoustics, Theatre Projects

Electrical Engineer: Ingeniør Per Rasmussen AS

Structural Engineer: Reinertsen Engineering

General Contractor: 55 contracts

Category: Opera House

Geological Engineer: NGI

Artists, Integrated Artwork: Kristian Blystad, Kalle Grude, Jorunn Sannes, Astrid Løvaas og Kirsten Wagle

Client: Ministry of Church an Cultural Affairs

Theatre Consultants: Theatre Projects

City: Oslo

Country: Norway

About the Building Client and the User. Statsbygg is Norway's largest civil property manager, with 650 employees. It is the state's main consultant on building and property issues, development and management. Statsbygg is a management company under the Ministry of Renewal and Administration, but provides services and support to all ministries and state organs.
In 1998 the National Assembly decides that Statsbygg would be the building client for the new operahouse, responsible for planning and management. Statsbygg procure services in the private sector, but are responsible for professional coordination and quality control of the consultants, contractors and suppliers.
The Norwegian Opera and Ballet is the building's end user. They are Norway's largest music and theatrical institution. Their core purpose is to be the national producer of opera, ballet, music and dance theatre, and concerts. They intend to have approx. 300 shows and 250,000 visitors per year. The Operahouse will be a workplace for approx. 600 employees from more than 50 professions.

The operahouse is the realisation of the winning competion entry. Four diagrams, which were part of the entry, explain the building's basic concept. "The wave wall:" Opera and ballet are young artforms in Norway. These artforms evolve in an international setting . The Bjørvika peninsula is part of a harbour city, which is historically the meeting point with the rest of the world.. The dividing line between the ground 'here' and the water ‘there'is both a real and a symbolic threshold. This threshold is realised as a large wall on the line of the meeting between land and sea, Norway and the world, art and everyday life. This is the threshold where the public meet the art.

"The Factory." A detailed brief was developed as a basis for the competition. Snøhetta proposed that the production facitities of the operahouse should be realised as a self contained, rationally planned ‘factory'. This factory should be both functional and flexible during the planning phase as well as in later use. This flexibility has proved to be very important during the planning phase: a number of rooms and romm groups have been adjusted in collaboration with the end user. These changes have improved the buildings functionality without affecting the architecture.

"The Carpet." The competion brief stated that the operahouse should be of high architectural quality and should be monumental in it's expression. One idea stood out as a legitimation of this monumentality: The concept of togetherness, joint ownership, easy and open access for all. To achieve a monumentality based on these notions we wished to make the opera accessible in the widest possible sense, by laying out a ‘carpet' of horizontal and sloping surfaces on top of the building. This carpet has been given an articulated form, related to the cityscape. Monumentality is achieved through horizontal extension and not verticality. The conceptual basis of the competition, and the final building, is a combination of these three elements - The wave wall, the factory and the carpet.

Urban situation. The operahouse is the first element in the planned transformation of this area of the city. In 2010 the heavy traffic beside the building will be moved into a tunnel under the fjord. Due to its size and aesthetic expression, the operahouse will stand apart from other buildings in the area. The marble clad roofscape forms a large public space in the landscape of the city and the fjord. The public face of the operahouse faces west and north - while at the same time, the building's profile is clear from a great distance from the fjord to the south. Viewed from the Akershus castle and from the grid city the building creates a relationship between the fjord and the Ekerberg hill to the east. Seen from the central station and Chr. Fredriks sq. The opera catches the attention with a falling which frames the eastern edge of the view of the fjord and its islands. The building connects city and fjord, urbanity and landscape.To the East, the ‘factory' is articulated and varied.One can see the activities within the building: Ballet reheasal rooms at the upper levels, workshops at street level. The future connection to a living and animated new part of town will give a greater sense of urbanity.

Collaboration with artists. For Snøhetta, close collaboration with artists has always been an important part of building projects. As early as the competition stage, artist were invited in as collaborators, and the wished to continue this from the beginning of the design phase. Snøhetta have tried to avoid having artist apply ‘decoration' to the architecture, prefering to allow for an open dialogue between artists, artisans and professionals with various approaches to important building elements. With the operahouse, the architect intended that both the large marble clad roofscape and the aluminium clad facades should be approached as collaborative endeavours.An early collaboration was established for the sone roof with artists: Kristian Blystad, Kalle Grude og Jorunn Sannes. One year later, in accordance with the guidelines for state funding building projects, a committee for integrated artwork was established. This committee engaged the artists Astrid Løvaas og Kirsten Wagle to collaborate on the design of the metal cladding elements.

Choice of materials. The materials, with their specific weight, colour, texture and temperature, have been vital to the design of the building. Snøhettas architecture is narative. It is the materials which form the defining elements of the spaces. It is the meeting of the materials which articulates the architecture through varied detail and precision. In the operahouse, three main materials were specified as early as the competition entry: White stone for the ‘carpet', timber for the ‘wave wall', and metal for the ‘factory'. During the continued work on the project, a fourth material, glass, which allows for the exposure of the underside of the ‘carpet', has been given specific attention.

Stone. After an international tender competition, th italian marble, La Facciata, was chosen. This is a stone which, in common with other marbles, retains its brilliance and colour even when wet. It has the necessary technical quality in terms of stabitity, density, and longevity. The producer, Campolonghi, has had the professional ability, capacity, and experience necessary for such a large and complex project. The accessibble area of the ‘carpet' is approx. 18,000 m2. Its detailed design has been important: the architect desired that it should not interfere with the general dorm of the building but that it simultaneously was articulated enough to be ineresting at close quarters. Together with the artists several alternatives were proposed before a particular non repetitive pattern with integrated riased areas, special cuts, various surface textures, and specific details were designed to articulate the main geometry.

Timber. Oak has been chosen as the dominating material for both the 'wave wall' and the main auditorium. For the wave wall it has a light and varied surface. Oak is used throughout for the floors, walls and ceilings. The wave wall has a complex organic geometri made up of joined cone shapes. It is also an important acoustic attenuator within the foyer space. To achieve these goals it is made up of smaller elements which can deal with the changing geometry and provide acoustic absorption.
Inside the auditorium oak has been chosen for a number of reasons: It is dense, easily formed, stable and tactile.
The oak has been treated with amonia to give a dark tone. Here too oak is used for floors, walls, and ceilings, as well as balcony fronts, and acoustic reflectors.

Metal. An operahouse is designed and built to have a long lifespan. This means that a simple, modern metal cladding, such as we associate with factories and workshops, needs to be re-evaluated and redesigned. After a consideration of aesthetics, longevity, maleability and the possibility to make very flat panel, aluminium wa chosen. To give the panels further quality, a collabarative process was begun with two artists. The design team initially aimed for an industrial modulrity but that the panels themselve should have greater visual quality. The panels were punched with convex spherical segments and concave conical forms. The pattern was developed by the artists based on old weaving techniques. In all, eight different panels were designed which give a constantly changing effect depending on the angle, intensity and colour of the light playing on them.

Glass. The high glass facade over the foyer has a dominant role in the views of the building from the south, west, and north. Early in the project it was realised that this glass faced was more important than previously assumed, both during the day and night when it would act as a lamp illuminating the external surfaces. The glass façade is up to 15 meters high. It was the architects intention to design a glass construction with an absolute minimum og columns, framing, and stiffening in steel. The solution was to use glass fins where minimised steel fixings are sandwiched inside the laminates. The requirements for the glass's stiffness increased due to the desire for large panels and slim joints where the panels meet. Thick glass of this sort tends to be quite green rather than transparent. It was therefore decided that the façade of the operahouse would use low iron glass.

Plan solution, general arrangement. The building is split in two by a corridor running north-south, the ‘opera street'. To the west of this line are located all the public areas and stage areas. The eastern part of the building houses the production areas which are simpler in form and finish. Comprising 3 to 4 storeys above ground. There is also a basement level - U1 - below this part of the building. The sub stage area is a further 3 storeys deep.

The building's western part. A marble clad plaza leads the visitors to the foyer and other public areas. A secondary entrance on the north façade gives firect access to the restaurant and foyer. To the south, the foyer opens up to the inner oslo fjord and views of Hovedøya island. To the west and north it is views of the city which dominate, while the auditoria lie to the east. There can be as many as 1900 audience members in the building. 1400 in the main auditorium, 400 in stage 2 and 150 in rehearsal room 1, which doubles as a black box theatre. There is a brasserie to the south of the foyer, a restaurant to the north and several bars which can be run separately from the performances. Service functions such as education spaces, cloakrooms, toilets, information/ticketing desk and diverse smaller rooms are located around the foyer. From the foyer, at ground level, and from the public galleries, access is provided to the two main auditoria. The large stage area occupies a significant part of the building footprint. Here is the main stage (16m x16m) with an 11.8m deep substage, two side stages and two rear stages, as well as a scenery hall and store. There is a free height of minimum 9m throughout these areas. Storage for the backdops is located above the rear side stage. Finished scenery for several performances and acts can stand ready on the rear and side stages as well as below stage. In addition, the large rehearsal room is located in direct connection to the stage areas and can provide further scenery storage should this be necessary. The orchestra rehearsal room - an acoustically sensitive space - is also located in the western part of the building - at basement level. This hall is the orchestra's most important rehearsal space and can also be used for recording purposes. The requirement for variable acoustics is achieved by the use of adjustable panelling and drapes. The room can achieve similar acoustics to the main auditorium.A passage from the foyer, along the southern façade, leads to rehearsal room 1 allowing it to be used as a public performance space.

The building's eastern part. To the east of the ‘opera street' are located all the production and administration areas; approx 1000 rooms of varing size and function. The opera street is the main communication artery for all the employees - almost 600 persons from more than 50 professions.

A large loading dock running east west splits the back of house area in two. Here also, the dimensions of the space are given by the size of scenery elements, up to 9m high. To the north lie the ‘hard workshops' where the scenery is made. Several professions have their workspaces here, carpenters, metalworkers, painters and decorators. The finished scenery is moved through the loading dock and directly into the rear stage area. To the south lie all the other functions necessary to serve the needs of the dancers and singers: ‘Soft workshops' with costume production, wigs, hats, gloves and make up areas. Also administration and changing rooms are located here.A spaceous green courtyard is at the heart of this area on levels U1 and Ground.Most of the changing rooms house 4 performers with all the necessary costume and make up for each show. The rooms are also intended to be a place for relaxing and concentrating and are therefore equipped with a day bed.
The opera and ballet departments have several large rehearsal rooms in this zone on levels 3 and 4. and it is possible to transport scenery from the loading dock to the rehearsal spaces on level 3 via an elevator with a clear height of 6m. The largest of the rehearsal rooms has a clear height of 9m and is as large as the main stage. This allows the dancers to practice a complete performance. All these spaces have walls with acoustic attenuation. There are also a number of small rehearsal rooms at palan 2. The wig makers, make up artists, and dressers are located closer to the stage at level 1 (ground). From here the artistes can access the stage areas at ground level or from the basement. At level 2 there are msic archives, offices, and support functions for the orchestra as well as a health center and gym. Level 3 houses the administration department with a large canteen wo the south with a terrace overlooking the fjord.

The main auditorium. The main auditorium is a classic horseshoe theatre built for opera and ballet. It houses approx. 1370 visitors divided between stalls, perterre, and three balconies. Technical spaces occupies the area above balcony 3.
The orchestra pit is highly flexible and can be adjusted in height and area with the use of three separate lifts. On each side of the stage are mobile towers which allow for adjustments in the proscenium width for ballet or opera without damaging the acoustics of the hall. Reverberation time is fine tuned using drapes along the rear walls and control rooms for sound and light are located to the back of the hall. The form of the auditorium is based on several relationships: short distance between the audience and the performers, good sight lines, and, above all, excellent acoustics. The architectural intentions for a modern auditorium with traditional, acoustic musical performance have been developed in parallel with requirements for visual intimacy and acoustic excellence. In older opera halls acoustic attenuation was often achieved by using rich decorative, sculptural elements on most surfaces. In this case the requirements have been met with a clean, carved aesthetic using a modern formalistic language. The requirement for a long reverberation time results in a room with a large volume. In this case the volume is increased by the use of a technical gallery which cantilevers out over the walls below, giving the hall a T shaped section. The main structure of the stone clad roof above is included in the volume of the hall rather than being hidden behind a false ceiling.

Optimum acoustics have been achieved by the following methods:
• We have, also as an aesthetic move, given the balcony fronts a geometry which changes relative to its location in the room and the acoustic function necessary in each location. At the sides the form reflects sound back down to the audience whilst at the rear it sends soungs in multiple direction to avoid focussing.
• The oval ceiling reflector visually finishes the hall and also reflects sounds in very specific ways. The same principle is used as with the balcony fronts.
• The rear walls at each level are made up of convex panels to avoid focussing and to spread sound evenly through the room.
• The geometri of the interlying walls, main orchestra reflector, and the mobile towers are modulated to scatter sound around the space. Using timber staves of varying dimensions to modulate sound of different wavelengths.
• All the surfaces are of relatively dense materials to avoid high frequency vibrations. Balcony fronts are 50mm solid oak where the rear wall panels are 100mm MDF with oak veneer.

The double curvature of the balcony fronts and oval ceiling ring are made of pre-fabrcated oak elements made of solid stave glued together, amonia treated and the routed from 3D computer drawings before oiling and polishing. The dark coluour is particularly suited to the theatre space and the oak gives a rich, warm, and intimate feel to the space.
The seats are designed to absorb as little sound as possible. Materials are dark timber and a specially design orange red fabric as a counterpoint. Text display screens are built into the seat backs so that the audience can individually choose to read the libretto in a number of languages.The auditorium is illuminated by a snøhetta designed chandelier in the form of a shallow lens as well as inbuilt LED fittings in the cailigs and floor.

The chandelier. The chandelier, which is suspended inside the oval reflector, is an important element in the hall as performs several tasks. It is the auditoriums main source of illumination, using LED for the first time in such a setting. It weighs 8.5 metric tons and has a diameter of 7m. It is made up of 5800 hand cast glass crystals through which 800 LED lights shine. This bathes the room in a cool diffuse light. The whole chandelier can be lowered to the floor for maintenance. It is also an important acoustic reflector. This explains the particular form which scatters and diffuses sound. The distance between the strips of crystals increases towards the stage to allow a greater amount of sound to pass through and therefore contribute to the reverberance of the space. It is placed forward of the centre in order to allow unhindered sightlines from the follow spot room on the rear of the oval reflector.Finally it forms a visual closure to the hall itself to take attention away from the technical spaces and structure above.

The stage curtain. The stage curtain is also an important element in the auditorium. Together with the chandelier and seat fabric it is a contrast to the dark timber. It has been made by the american artist Pae White, following an international competition. She has worked with digital images of aluminium foil which reflects and adopts the colurs of the auditorium. These images are then transferred to a computer driven loom.

Stage 2. Stage 2 can, depending on the chosen seating configuration, house an audience of up to 400. It will be used by both opera and ballet, as well as for banquet functions, rock concerts, experimental performances and children's theatre. It is a multi use hall where the seats, which are on large wagons, can be repositioned in a number of different configurations. There are 2 large elevators which form an amphitheatre, orchestra pit and transport seating wagons for storage in the basement. The area which is normally the stage is made up of removable floor elements. The auditium has no flytower but rather an extensive motorised pully system to hang and transport scenery, backdrops and acoustic reflectors when necessary. A 9m high sliding gate connects the stage area with the back stage zones and scenery stores. The reverberation time in the hall can be damped down for amplified performances. The client required an auditorium with the flexibility of a black box but with an amount of architectural quality and identity. These to requirements are generally considered to be mutually exclusive, but after close discussions with the end user, a solution was found where of a black box has a high quality cotrasting, freestanding structure placed inside it. This ‘object' has rounded, high gloss, red paneling on the outside and a cooler metallic silver finish in towards the stage. Four technical bridges span across the space at high level housing lighting and ventilation and forming an important visual and acoustic ceiling. Between the columns, large, black painted doors and removable panels are used to adjust to different configurations. These panels have also been given acoustic consideration.

Interiors. The exterior of the operahouse becomes diffuse as night falls. The large timber ‘wave wall' in the foyer is illuminated and the building takes on a completely different character. The interior becomes the façade. It shows how interdependent the interior and exterior of the building are. The building's architectonic ideas and concepts have also been used in the buildings interiors. The task has included considerable interior planning based on the schedule of rooms, functions, colours, materials, and surface treatments, coordinating lighting schemes, technical instalations, built in furniture, wet rooms, kitchen solutions, elevator cars, fittings and fixtures. It has also encompassed design and coordination of the end user equipment and loose furnishings. Cooperation between the various architectural diciplines has been vital.

Interiors concept, public spaces. The experience of the buildings exterior clearly requires that the interiors be of equally high architectural quality. On entering the building one is first lead in under the lowest part of the sloping roofscape. Where the ceiling falls to meet the floor. This area is used for the public cloakroom where a copse of slim columns hold the visitors coats. Further out into the foyer, four volumes hold up the roof. The perforated, illuminated cladding of these is another example of integrated artwork. In this case by Olafur Eliasson. These white form house the public toilets. Moving out into the open space of the foyer one is below an expance of white, sloping cailing held up by angled white columns which meet in clusters at floor level. The grand staicase is peeled out of the wooden wall and leads up to 3 galleries around the auditorium. Thus providing access to the upper levels of the hall. The interior of the wooden wall has an intimate feel in contrast to the open, white foyer. Dark light locks lead the audience into the heart of the building. The main auditorium. The feeling is of being inside a carved out piece of timber, or perhaps within a musical instrument.

Furnishings and equipment in the public areas. The interior, from cloakroom to auditorium can be described as a formalistic journey which takes the visitor from the open unknown to the enclosed and secure. The level of abstraction to be seen in the outer spaces ahs made it natural to minimise the number of recognisable builing elements and details. At the same time it has been a clear aim that the furniture elements use the same design language as the building as a whole. Larger elements such as bar counters, shop fittings, ticketting desk, and cafè interiors are either integrated in larger building forms or designed as free standing sculptural forms in white corian. These can be completely closed down when not in use. The cloakroom and foyer are further furnished using simple seating forms and high tables made of steel plate coated with industrial rubberized black lacquer. Upholstery is with flat sheets of felt and sheep skin. Signage is made of the same black steel and white glass surfaces complete a number of the interior elements.

Furnishing of the production areas. These zones are designed around eronomics, functionality, and experience. The harde workshops are rational rooms where the logistics of mechanisation dominate the design. Wig and make-up workshops have been provided with specially designs workstation modules specific to the user's requirements. The costume department, which is a hectic space full of activity has been given solution specific to its complex logistics. The three artforms; Ballet, opera, and orchestra all have their requirements for changing rooms fulfilled with standardised but purpose built furniture. However, the orchestra have larger, 10 man changing rooms with areas for unpacking instruments, rest, and changing prior to a performance, amd with shared access to toilets and showers. The Ballet, choir and soloists have smaller 4 or 6 man rooms with person specific places and showers shared with the neighbouring room. For the ballet these rooms function as a home base in a day filled with training and rehearsals.All the changing rooms are specially design with fitted, standardised furniture, make up tables, day beds and cupboards.A great deal of work has gone into designing the rehearsal spaces for the different groups. These are important working spaces, with optimised acoustics, ventilation, and lighting. The intention has been to provide a great deal of spacial quality and the acoustic wall panelling in particular contributes to this.

Material use in the production areas. The brief for these area specifies that they should be simple and inexpensive. This means that they are of general office quality with painted walls and linoleum flooring. This works well as a neutral background to the opera's colourful costumes and stage elements which enliven the spaces.The colour palette is therefor quite simple and neutral. The open courtyard forms a central reference point to the production areas and the corridors which encircle it are given a dark colour to make orientation easier. The rehearsal rooms have different characters for ballet, opera and choir.. The ballet spaces are light and airy with views over the fjord to the south. Whilst the choir space is more introvert with daylight from a high clerestory window facing east. Enclosing the musical experience. Colours and materials have a warmer, darker hue.

Furniture and end-user equipment. Even though the building is large, there is little loose furniture. We have attempted to to simplify and standardise the choice of furniture and relate it to the building's architectural design. A small number of quality furniture producers have been chosen and the pieces have a clean, contemporary design which is simultaneously classical enough to be timeless.  Landscaping. The opera's landscape comprises of the marble roof, additional marble clad areas, and the areas between the building and the surrounding streets. Access to the plaza and the main entrance is over a marble clad footbridge over the opera canal. The plaza forms a part of a public promenade and cycle lane which continues around the west and south sides of the building, and eventually coming to a planned bridge over the Aker river to the east. As early as the competition entry, Snøhetta proposed that the roofscape should be openly accessible to the general public and that it should be clad with white stone. Today the building's defining feature is the characteristic geometry of the roof as it rises from the fjord and is laid out like a carpet over the public areas. An important move has been to introduce channels along the roof edges with ramps and steps. This allows the integration of regulation height balustrades with raising the line of the roof itself.

To achieve enough acoustic volume in the auditorium, the roof has been raised independently inside the line of the balustrades. This has created a new viewing point from which the city and the fjord can be experienced. The roofs are mostly too steep for wheelchair use but access to the near flat, upper areas is provided via a dedicated elevator. The surface treatment of the stone, its pattern, cuts and lifts which create a shadow play, have been designed in close collaboration with the artists. The white marble is ‘La Facciata' from the Carrara quaries in Italy. The north facede and all the stone cladding which is in contact with water is a norwegian granite called 'Ice Green.' Prototypes and tests at full scale were studied at the contractor's facitities before the final choices were made for colour nuance and surface texture. A running quality control regime has been implemented throughout the production process. Adjacent areas During the building period it became clear that rapid and considerable settling of the ground level around the building would need to be addressed.

Large areas of gravel which is designed to take local vehicular traffic have been laid around the building footprint. This is easy to adjust as the ground sinks relative to the building which is founded on the bedrock. Trees are planted in the gravel areas, and a zone of street furniture is located along the pavement line with cycle parking, benches and specially designed streetlamps in stainless steel. The pavements are of asphalt with black granite edges and larger areas of granite paving wto highlight the entrances to the restaurant, opera street, and stage entrance. The dark grey colur palette is a clear contrast to the light stone and aluminium of the building itself within a cool monochrome language. Landscaping of the surrounding areas has been designed in collaboration between Snøhetta and Bjørvika Infrastructure who have been responsible for the planning of the street around the operahouse.

Courtyard. The courtyard is a garden in the middle of the production area of the building. Surrounded by facades of black glass, aluminium and timber and open to the sky. There is direct access to the courtyard from ground and basement levels while the upper levels experience it as a green lung deep inside the building. In front of the sound-insulated rehearsal rooms at basement level, vegetation has been planted to form a screen. The floor of the courtyard is a composition of timber dekking, white marble, and green areas. A marble cladd stair connects the two levels. Grasses, climbing plants and perennials are planted around clusters of cables reaching up to the upper levels and providing shade to the facades.

- Data from Snøhetta

# Norwegian National Opera and Ballet
Giving waterfront space back to the public

2000–2008

Architecture, Art, Landscape Architecture, Interior Architecture

The opera house is part of Oslo's revitalization strategy to redevelop the city's historically industrial waterfront into an active public space and was the first building to be completed in the Bjørvika area. It is built on piles in the Oslo fjord, abutting reclaimed land that extends Oslo's harbor, giving more of the city's waterfront space back to the public.

Inside, its horseshoe-shaped main hall is reminiscent of classical theaters of the past, providing a stage for wide-scale world-class performances. Outside, its open plaza and sloping, walkable roof make the building as much landscape and public space as architecture, making it a local landmark and a destination also for non-opera and ballet audiences. Generous windows at street level give the public a glimpse of the scenery workshop activities, and with eight connected art projects, making it one of Norway's largest public art projects, visitors are invited to awareness and engagement.

2. The right to roam

An ancient custom now protected by Norwegian law, allemannsretten (Norwegian for the right to roam) ensures one's freedom to move unrestricted throughout the countryside. Applying this idea to the interior in addition to the exterior of a building that is, to the landscape and also to the architecture - removes the oppositional nature from these terms. As they become synonymous, interior and exterior spaces become continuous. It was in this spirit that our proposal for the Norwegian National Opera and Ballet was conceived.

3. Keyless structure

Like the roof plaza, the lobby of the Oslo Opera House is keyless; visitors have been free to roam its spaces at all hours since the building opened in 2008. White carrara marble seamlessly covers the roof and lobby to denote a continuous, public ground plane, also a 20 000 m2 art piece in itself. Today, the public roof plaza routinely hosts outdoor concerts and simulcasts of operas happening inside the main theater, with as many as 15,000 people attending a single rooftop event. Similarly, the lobby, with its theater-quality acoustics, serves as a venue for public performances.

4. Oslo's most popular tourist attraction

The Opera House has made good on its promise to serve as the keystone of Oslo's waterfront redevelopment, including the Munch Museum, the National Museum, and the Oslo Main Library - heralding further expansion of the cultural district. Welcoming 1.7 million visitors annually, the Opera House has solidified its role as an economic driver for both the neighborhood and the city.

5. Cleaner fjord

Construction of the Opera House has also produced other benefits. The large-scale environmental cleanup of the site, once a highly polluted industrial shipyard, has restored the Oslo Fjord to its cleanest in nearly 100 years. Healthy enough for humans and animals alike, the waters now support several active public swimming areas and more than forty returning plant and animal species. The success of this cleanup has led to national efforts to revive the coastline, deepening the impact of the Opera House beyond the scope of Oslo's masterplan and expanding the boundaries of one's right to roam well beyond the roof plaza, to the outer limits of the Norwegian shore.

6. Eight art projects and 17 artists

The National Opera and Ballet in Oslo is also one of Norway's largest public art collections, featuring eight art projects and seventeen artists. Integrated art defines much of the design. The walkable marble roof is designed by the artists Kristian Blystad, Kalle Grude and Jorunn Sannes, while the textile stage curtain was created by visual artist, Pae White. 

The perforated cladding in the lobby for the bathrooms was designed by the artist Olafur Eliasson. Chosen through an international art competition, the project allows the visitor's perception of the lobby areas to change over time and with movement around the structures. A soft light changes color while the diamond-like filagree shows differing characteristics of scale and hue when passing alongside the walls.

7. Interior

The interiors have been designed by Snøhetta to provide an integrated and complimentary quality to the informal and fluid forms of the public areas. The seating areas are arranged to promote comfort while the furnishings themselves are flexible and modular allowing for change over time.

Award

- Achieved Landmark Status for Historic Preservation2012

- The Houen Foundation Award2012

- International Architecture Awards, Chicago Athenaeum2010

- Marble Architectural Awards2010

- European Prize for Public Space2010

- World Architecture Award, Best Cultural Building2009

- Statens Byggeskikkpris2008

- World Architecture Festival, Display2011

- Mies van der Rohe Award2009

- Data from Visit Norway

Part landmark, part public space, part backstage world – Oslo's Opera House is designed to be explored.

### The Opera House

Home to The Norwegian National Opera & Ballet

Designed by Snøhetta

Opened in 2008

Cost: NOK 4.3 billion

Around 1,100 rooms

Around 600 employees across more than 40 professions

## Enjoy free seating

The Norwegian nature is free for everyone to walk in, and The Oslo Opera House, which opened in 2008, was built as an extension to this idea.

Usually, you are likely to be arrested if you walk on rooftops. This new building in the very epicentre of the capital of Norway feels like the complete opposite of the usual “Please don’t touch” culture tourists are often met with worldwide. The subtle variations in the structure of the marble-embellished roof is signed by Norwegian artists Kristian Blystad, Kalle Grude and Jorunn Sannes, and is truly a beautiful surface meant to be stepped on.

Please keep in mind that under your feet there are three highly differently designed scenes, a myriad of public rooms and halls to explore, and a vibrant workplace for more than 600 opera and ballet professionals.

This structure made for walking also offers an unexpectedly cool sit-down experience. The innovative physique of the Opera House with its marble-covered roof will unveil surprisingly different angles of the city you have come to visit.

On your right: The fjord archipelago with small, traditional wooden summer houses painted in bright, optimistic colours. To your left: A quick scenery over the city core of Oslo. And further behind: The many fairytale-like shaped hills and mountains which embraces Oslo, the fastest-growing capital in Europe.

Throughout the year, The Oslo Opera House hosts a large variety of outdoor plays, concerts and other events. As a bonus, the structure of the building has given the waterfront back to citizens and visitors.

## Enter the playroom

Make an early arrival before the play begins and guests commence to fill the hall. Explore that special silence of great expectations that occurs before it all starts. The for an Opera House unusual wooden theme is a soft reminder of Norwegian nature and history.

The intimate horseshoe shape of The Main Hall hints to the origins of classic theatres. The design of the theatre’s main curtain is signed by artist Pae White.

## Full moon experience

The central chandelier is created of hand-cast glass bars lit from behind by LED lights. When lit or turned of, it has a moon-like presence.

## Get close to the stars

The often massive media attention created by the architecture of the Opera House has helped attracting internationally renowned opera and ballet performers to Oslo. The ever changing ensemble constantly consists of a fruitful combination of highly acclaimed national and international performers.

The purposely set intimacy of the theatre layout enables you as a guest to get close to the performing stars of which you have come to experience.

## Stretch your imagination

A classical piece of opera or ballet should stay true to the origins of its interpretation, if you ask some. Others would like to see a brave new take on it. In this house, you will find both, and since the opening in 2008, The Norwegian National Opera & Ballet has been known for its variety.

## Get lost

The Opera House has a huge inner space with spectacular designs, that is open to the public. In order to create unusual surfaces, architects at Snøhetta worked with artists. One of many examples of astonishing design by different artists is the perforated cladding created by artist Olafur Eliasson.

## Your turn to play

Come visit the building that was not particularly inspired by previously built landmarks, but rather by Norwegian nature.

People from all over the world have come to walk on the roof and to experience various performances — now it’s your turn to come here and play.

- Data from Dezeen

Snøhetta's Oslo Opera House was the most significant building of 2007

Amy Frearson | 13 January 2025  17 comments

The next instalment in our 21st-Century Architecture: 25 Years 25 Buildings series is Oslo Opera House, a Snøhetta-designed project that pioneered the concept of architecture as landscape.

The home of the Norwegian National Opera and Ballet has become one of Oslo's most iconic buildings, primarily because of the sloping public plaza that forms its roof.

The project kickstarted a wave of buildings with publicly accessible roofscapes and became the calling card for Snøhetta, an emerging architecture studio with big ambitions.

Snøhetta won the competition to design the Oslo Opera House in 2000, just as construction was completed on its first major commission, the Bibliotheca Alexandrina library in Egypt.

The studio takes its name from the tallest peak in Norway's Dovrefjell mountains. Its ambition from the start was to make landscape design more integral to the architectural process.

Founding partners Kjetil Trædal Thorsen and Craig Dykers saw an opportunity to push this concept even further with the opera house than they had with the library.

While the library was designed to merge with the landscape, the opera house was conceived as a landscape itself.

"We were seeing at the time that construction projects didn't have budgets for their outdoor areas," said Thorsen in an interview with Dezeen in 2019.

"We said this can't be the case, because we're neglecting our public space," he told Dezeen. "This is how we ended up at the Oslo Opera House, where there is no differentiation between public space, building and site."

The building achieves this with a wedge-shaped form, built on the edge of the Oslofjord in the city's former docklands.

Today this part of the city is home to many impressive buildings – examples include the Barcode Project office blocks, the Deichman Bjørvika library and the new Munch museum – but it was largely derelict at the time.

The building slopes down at its western end, allowing the groundscape to extend from the water's edge to the top of the roof. Snøhetta described it as "a carpet of horizontal and sloping surfaces".

These surfaces are clad with Norwegian granite and Carrara marble, enhancing the sense of a snow-covered hill.

The opera house's interior is just as impressive as its exterior. In contrast with the cool-toned exterior cladding, oak was the material of choice for the auditorium.

This horseshoe-shaped concert hall resembles a giant drum, rising through the middle of the generous atrium.

A powerful and beautiful statement, radiant with music and songJonathan Glancey in The Guardian

Baltic oak clads the "wave wall" that wraps this drum and the walls, floors, ceilings and balcony fronts inside the auditorium. Some of this wood was treated with ammonia, which gives it a darker tone.

The hall also includes a contemporary chandelier with 5,800 hand-cast glass crystals and 800 LED lights, providing a lighting spectacle overhead.

Oslo Opera House was completed in 2007, which is when the city's residents were first invited to climb the roof. It officially opened in April 2008 with a ceremony attended by King Harald.

It was an instant hit with critics, who applauded the way it gave back to the city without compromising the building's primary function as a world-class concert hall.

"The Oslo opera house is a powerful and beautiful statement, radiant with music and song, one that announces Norway's arrival as a cultural centre," said Jonathan Glancey in The Guardian.

The opera house was awarded the biennial European Union Prize for Contemporary Architecture, the Mies van der Rohe Award, in 2009. It also won the European Prize for Urban Public Space in 2010.

Francis Rambert, chair of the Mies van der Rohe Award jury, described it as "more than just a building".

"It is first an urban space, a gift to the city," she said. "The building can be considered a catalyst of all the energies of the city and is emblematic of the regeneration of its urban tissue."

The true indicator of the building's success is its popularity.

In the summer, the roof is usually flooded with skateboarders, swimmers and sunbathers, and often hosts outdoor performances. It's also well-used in the icy winter months when it becomes a destination for sledging.

In this way, Snøhetta achieved its aim of creating a truly democratic building, enjoyed by more than just those who enter inside to watch the opera.

Did we get it right? Was Oslo Opera House by Snøhetta the most significant building completed in 2007? Let us know in the comments. We will be running a poll once all 25 buildings are revealed to determine the most significant building of the 21st century so far.

This article is part of Dezeen's 21st-Century Architecture: 25 Years 25 Buildings series, which looks at the most significant architecture of the 21st century so far. For the series, we have selected the most influential building from each of the first 25 years of the century.

The illustration is by Jack Bedford.